SMART HELMET FOR COAL MINERS

A Mini-Project Report submitted

In partial fulfillment for the Degree of

B. Tech

in

Electronics and Communications Engineering

By

R.ARAVIND      (20241A04N0)

K.THARUN KUMAR (20241A04L2)

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

GOKARAJU RANGARAJU INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

BACHUPALLY, KUKATPALLY

HYDERABAD-500090

2022-2023

Gokaraju Rangaraju Institute of Engineering & Technology

Department of Electronics and Communication Engineering

CERTIFICATE

This is to certify that this thesis entitled “SMART HELMET FOR COAL MINERS” submitted by K.Tharun Kumar (20241A04L2), R.Aravind (20241A04N0) in partial fulfillment of the requirements for the degree of Bachelor of Technology in Electronics and Communication Engineering of JNTUH, during the academic year 2022-23, is a bonafide record of research work carried out by his/her under our guidance and supervision. The contents of this thesis, in full or in parts, have not been submitted to any other university or Institution for the award of any degree or diploma.

DR.V HIMABINDU

Internal Guide			External Examiner			Head of the Department

DECLARATION

I  hereby declare that this mini-project report title “SMART HELMET FOR COAL MINERS” is the work done during the period from December 2022 to May 2023 and is submitted in partial fulfillment of the degree of B. Tech in (Electronics and Communications) is a record of original work carried out by us and has not formed the basis for the award of any other degree or diploma, in this or any other Institution or University. In keeping with the ethical practice in reporting scientific information, due acknowledgement have been made wherever the findings of others have been cited.

R.ARAVIND       (20241A04N0)

K.THARUN KUMAR (20241A04L2)

ACKNOWLEDGEMENT

I gratefully thank my project guide ,Department of Electronics and Communication Engineering, Gokaraju Rangaraju Institute of Engineering and Technology for her excellent guidance and timely suggestions throughout the project work.

I wish to express my sincere gratitude to Department of Electronics and Communication Engineering for providing valuable guidance and suggestions during the course of work.

I wish to express my sincere thanks to Department of Electronics and Communication Engineering, Gokaraju Rangaraju Institute of Engineering and Technology for reviewing my work time to time and giving necessary suggestions needed during the project work.

I would like to convey my sincere thanks to Dr.HIMA BINDU VALIVETI, Professor and Head of the Department, Electronics and Communication Engineering, Gokaraju Rangaraju Institute of Engineering and Technology for valuable suggestions and motivation during the work.

I convey my deep sense of gratitude to Dr. J. PRAVEEN, Principal for the kind encouragement and support.

I would like to thank all teaching and non-teaching staff of the Department of Electronics and Communication Engineering for their kind support and suggestions which in turn helped me to complete the project successfully.

Finally, I would like to convey my sincere thanks to each and every person who have supported and helped me for the success of the project.

STUDENT NAMES

K THARUN KUMAR

R ARAVIND

ABSTRACT

Smart Helmet for Coal Mines Safety Monitoring project aims to improve safety conditions for coal miners by employing a smart helmet integrated with sensor technology and a mobile application. The project utilizes a combination of hardware and software components to monitor environmental parameters, detect potential hazards, and provide real-time notifications and alerts to miners and supervisors. The system incorporates sensors for gas levels, temperature, humidity, and motion, which transmit data to a mobile application via Bluetooth or a wireless communication module. The mobile app, developed for Android and iOS platforms, receives and analyzes the sensor data, provides real-time monitoring, and generates alerts and notifications based on predefined thresholds. Additionally, the app facilitates user authentication, allowing miners and authorized personnel to access the system securely. The Smart Helmet for Coal Mines Safety Monitoring project offers an innovative solution to enhance safety practices in coal mining operations and reduce the risk of aaccidents and injuries.

TABLE OF CONTENTS

| DESCRIPTION | PAGE NO |
| --- | --- |
| CERTIFICATE |  |
| DECLARATION |  |
| ACKNOWLEDGEMENT |  |
| ABSTRACT |  |
| CHAPTER-1 |  |
| INTRODUCTION 1.1PROJECT INTRODUCTION 1.2 AIM OF THE PROJECT 1.3 PROBLEM STATEMENT 1.4 RESEARCH OBJECTIVES |  |
| CHAPTER-2 2.1 BLOCK DIAGRAM |  |
| 2.2HARDWAREDESCRIPTION 2.2.1ESP32MICROCONTROLLER 2.2.2 DHT 22 SENSOR 2.2.3 MQ2 GAS SENSOR 2.2.4 BUZZER 2.2.5 OLED DISPLAY |  |
| CHAPTER-3 |  |
| 3.1 CIRCUIT DIAGRAM |  |
| 3.2 INTERFACING SENSORS 3.2.1 INTERFACING DHT22 WITH ESP 32 |  |
| 3.3 INTERFACING MQ2 WITH ESP 32 |  |
| 3.4 INTERFACING BUZZER WITH ESP32 |  |
| 3.5 INTERFACING OLED WITH ESP32 |  |
| CHAPTER-4 |  |
| 4.1 FIRE BASE |  |
| 4.2 STEPS TO ADD FIREBASE TO SENSOR DATA |  |
| 4.3 ESP32 STORE DATA TO FIREBASE DATA BASE |  |
| 4.4 KODULAR COMPANION |  |
| 4.5 ARDUINO CODE |  |
| CHAPTER- 5 |  |
| 5.1 FUTURE SCOPE |  |
| 5.2 CONCLUSION |  |
| 5.3 COURSE OBJECTIVES |  |

LIST OF FIGURES

| FIGURE | FIGURE NO |
| --- | --- |
| BLOCK DIAGRAM |  |
| ESP 32 MICROCONTROLLER | 2.2.1(a) |
| BLOCK DIAGRAM OF ESP 32 | 2.2.1(b) |
| PIN DIAGRAM OF ESP 32 | 2.2.1© |
| DHT-22 SENSOR | 2.2.2 |
| MQ-2 GAS SENSOR | 2.2.3 |
| BUZZER SENSOR | 2.2.4 |
| OLED DISPLAY | 2.2.5 |
| CIRCUIT DIAGRAM | 2.1 |
| CONNECTIONS/INTERFACING INTERFACINGDHT22WITH ESP32 INTERFACINGMQ2 WITH ESP32 INTERFACING BUZZER WITH ESP32 INTERFACING OLED WITH ESP32 | 3.2 3.3 3.4 3.5 |
| FIREBASE | 4.1 |
| KODULAR | 4.4 |
| RESULT | 4.5 |
| SPREAD SHEET | 5.1 |

CHAPTER - 1

INTRODUCTION

1.1 PROJECT  INTRODUCTION

India is a country, which is renowned for its extensive and distinct mineral reserves and big mining businesses. India produces about eighty eight minerals, out of which it has four minerals related to fuel, ten minerals that is of kind metals, fifty minerals that is of non-metallic in nature and remaining twenty four includes minor minerals

The mining industry, particularly coal mining, is known for its challenging and hazardous working conditions. In order to improve safety and protect the well-being of coal miners, the concept of a smart helmet has emerged as a promising solution. A smart helmet for coal miners integrates various advanced technologies to enhance safety, communication, and monitoring capabilities in the mining environment.

The smart helmet is designed to provide real-time monitoring of crucial parameters such as air quality, temperature, humidity, and presence of hazardous gases in the mine. It is equipped with sensors and communication modules that enable miners to stay connected with the surface control center and fellow workers. The helmet also incorporates features like built-in lights, cameras, and emergency alert systems to further enhance safety and situational awareness.

1.2 AIM OF THE PROJECT

The aim of the smart helmet for coal miners using ESP32 is to Enhance miner safety by integrating advanced technology into their protective headgear.. Detect and alert miners to hazardous conditions such as toxic gases and high levels of particulate matter. Monitor environmental factors like temperature, humidity, and air quality to prevent health hazards. Facilitate communication and collaboration among miners and surface personnel for improved coordination.Log real-time data for analysis to identify patterns, trends, and risks for better mine management.Overall, improve working conditions and minimize risks in coal mining environments.

1.3 PROBLEM STATEMENT

The problem statement for the smart helmet for coal miners using ESP32

Coal mining poses significant safety risks due to hazardous conditions such as toxic gases, particulate matter, and accidents. Miners lack real-time awareness of these risks, leading to potential health hazards and delays in emergency response.Current protective headgear lacks integrated technology to detect and alert miners to hazardous conditions and monitor environmental factors.Communication and coordination among miners and surface personnel are limited, hampering timely response and assistance in emergencies.The absence of location tracking makes it difficult to quickly locate miners in distress or those lost underground.Accurate detection of impacts or falls is lacking, resulting in delayed medical attention for injured miners. Data logging and analysis for risk assessment and mine management are inefficient or nonexistent.Overall, there is a need for a smart helmet solution using ESP32 to address these safety gaps and improve working conditions for coal miners.

1.4  OBJECTIVES

The research objectives for the smart helmet for coal miners using ESP32

- Evaluate the effectiveness of integrating ESP32-based sensors in detecting hazardous gases, particulate matter, and environmental conditions in coal mines.

- Assess the accuracy and reliability of the smart helmet's impact and fall detectioncapabilities using ESP32's built-in accelerometers and gyroscopes.

- Develop a robust two-way communication system using ESP32 to facilitate seamless communication between miners and surface personnel.

- Explore the usability and user experience aspects of the smart helmet, considering factors such as comfort, ergonomics, and ease of use for miners in challenging work environments.

- Evaluate the overall effectiveness of the smart helmet in improving miner safety, response times in emergencies, and coordination among team members.

Identify areas of improvement and propose recommendations for the future development and implementation of smart helmets using ESP32 technology in coal mining applications

CHAPTER- 2

2.1 BLOCK DIAGRAM

2.2 HARDWARE DESCRIPTION

2.2.1 ESP32 MICRO CONTROLLER

Fig 2.2.1(a) ESP 32 board

The ESP32 is a low-cost, low-power consumption, and versatile microcontroller It is a successor to the popular ESP8266 microcontroller and features more processing power, more memory, and additional features such as Bluetooth and WiFi connectivity.

The ESP32 microcontroller is based on a dual-core 32-bit processor that can operate at up to 240 MHz clock speed. It has a range of connectivity options including WiFi, Bluetooth, BLE, and Ethernet. The microcontroller also features a wide range of peripherals including GPIO pins, SPI, I2C, UART, PWM, ADC, DAC, and more.

The ESP32 is widely used in a variety of applications such as home automation, IoT devices, robotics, and wearable devices. It is also popular among makers and hobbyists due to its low cost, ease of use, and flexibility

SPECIFICATIONS

| PARAMETER | DESCRIPTION |
| --- | --- |
| Microprocessor | Tensilica Xtensa LX6 |
| Maximum operating frequency | 240MHz |
| Operating voltage | 3.3V |
| Analog input pins | 12-bit, 18 channel |
| DAC pins | 8-bit, 2 channel |
| Digital I/O pins | 39(of which 34 are normal GPIO pins) |
| DC current on I/O pins | 40mA |
| DC current on 3.3V pin | 50mA |
| SRAM | 520kB |
| Communication | SPI (4), I2C (2), I2S (2), CAN, UART (3) |
| Wi-fi | 802.11 b/g/n |
| Bluetooth | V4.2- Supports BLE and classic Bluetooth |

BLOCK DIAGRAM OF ESP 32

Fig 2.2.1(b) block diagram of esp 32

FEATURES OF ESP32:

Architecture: The ESP32 is based on the Xtensa LX6 CPU, which is a dual-core

processor with a clock speed of up to 240 MHz It also has an ultra-low-power co-

processor and inbuilt Wi-Fi and Bluetooth connectivity.

Wi-Fi and Bluetooth Connectivity: The ESP32 has inbuilt Wi-Fi and Bluetooth

connectivity, which allows it to communicate with other devices over the internet

and Bluetooth.

Peripherals: The ESP32 has several peripherals like GPIOs, SPI, I2C, UART, ADC,

DAC, etc., which allows it to interface with other devices like sensors, motors,

displays, etc.

Programming: The ESP32 can be programmed using various programming

languages like C++ , Python, and Lua. It also has an inbuilt IDE called ESP-IDF, which

makes it easy to program and develop applications.

Power Consumption: The ESP32 has low power consumption and can be

operated on a battery for a long time, making it ideal for IoT projects.

Overall, the ESP32 is a feature-packed microcontroller that is suitable for a

wide range of applications including IoT devices, robotics, wearables

Fig 2.2.1© Pin Diagram of ESP 32

2.2.2 DHT-22 Sensor

Fig 2.2.2 DHT 22

The DHT11 is a basic, ultra low-cost digital temperature and humidity sensor. It uses a capacitive humidity sensor and a thermistor to measure the surrounding air, and spits out a digital signal on the data pin (no analog input pins needed). Its fairly simple to use, but requires careful timing to grab data. The only real downside of this sensor is you can only get new data from it once every 2 seconds, so when using our library, sensor readings can be up to 2 seconds old. Compared to the DHT22, this sensor is less precise, less accurate and works in a smaller range of temperature/humidity, but its smaller and less expensive

PIN DESCRIPTION

| Pin of DHT11 | Description |
| --- | --- |
| VCC | Power supply |
| DATA | Input/output data |
| GND | ground |

APPLICATIONS

- Weather Monitoring:

- Home Automation:

- Indoor Garden Monitoring

- HVAC Systems:

- Food Storage:

- Industrial Applications:

- Internet of Things (IoT) Projects

ADVANTAGES :

- 1.Cost-effective: The DHT11 sensor is relatively inexpensive, making it an affordable choice for temperature and humidity monitoring projects.

- 2.Simplicity: The DHT11 is straightforward to use and requires minimal external components to operate. It uses a single-wire digital interface, making it easy to integrate into projects with microcontrollers.

- 3.Adequate accuracy: The DHT22 provides reasonably accurate measurements for most applications. It can measure temperatures within ±2°C and humidity within ±5%.

- 4.Low power consumption: The sensor has low power requirements, making it suitable for battery-operated devices or projects where power efficiency is crucial.

- 5.Easy availability: The DHT11 sensor is widely available from various suppliers, making it easily accessible for hobbyists and electronics enthusiasts.

FEATURES

- Temperature measurement

- Humidity measurement

- Single Bus-Interface

- Suitable for basic monitoring

2.2.3 GAS SENSOR

Fig 2.2.3 MQ 2 Gas Sensor

The gas sensor model MQ-02 is capable to detect various kinds of gases such as H2, alcohol, LPG, CH4 and smoke. It is very quick in responding to smoke. The figure depicts MQ-02 version of gas sensor.It generates voltage when gas enters its coil area. The voltage depends on gas concentration i.e. higher the gas concentration higher is the output voltage. This voltage is read by arduino or ESP32 board based microcontroller or MCU. Based on voltage measurement, appropriate action is taken as per programming.

FEATURES OF MQ-2 GAS SENSOR

➢ Operating Voltage is +5V

➢ Analog output voltage: 0V to 5V

➢ Digital Output Voltage: 0V or 5V (TTL Logic)

➢ Preheat duration 20 seconds

➢ The Sensitivity of Digital pin can be varied using the potentiometer.

➢ It can detect flammable gas in range of 300-10000 ppm

PIN DESCRIPTION

| GAS SENSOR(MQ2) | ESP32 |
| --- | --- |
| Supply | VCC |
| Ground | GND |
| Analog output | A0 pin |
| Digital output | Digital pins |

APPLICATIONS

- Gas leakage detection: The MQ2 sensor can be used to detect gas leaks in homes,

- commercial buildings, and industrial settings. It can quickly detect the presence of gas

- and alert users to potential hazards.

- Fire detection: The MQ2 sensor can detect smoke and other flammable gases, making

- it an essential component of fire detection systems.

- Air quality monitoring: The MQ2 sensor can be used to monitor the air quality in indoor

- and outdoor environments. It can detect harmful gases and pollutants in the air and

- provide data on the air quality.

- Industrial safety: The MQ2 sensor can be used in industrial settings to detect the

- presence of hazardous gases and to ensure worker safety.

- Smart homes: The MQ2 sensor can be integrated into smart home systems to monitor

- air quality, detect gas leaks, and alert users to potential hazards.

- Overall, the MQ2 sensor has a wide range of applications and is useful for detecting the

presence of various gases in the air.

- 2.2.4 BUZZER

- Fig 2.2.4 Buzzer Sensor

- A buzzer is an electronic device that generates an audible sound. They are commonly used in electronic devices such as alarms, timers, and warning systems.

- Buzzers can be divided into two main categories:

- Mechanical buzzers: These types of buzzers use a mechanical vibrating element (such as a diaphragm) to create sound. They are typically louder than electronic buzzers and have a longer lifespan but they require a DC voltage to operate and can consume a lot of power.

- Electronic buzzers: These types of buzzers use an oscillating electronic circuit to create sound. They are typically smaller and less expensive than mechanical buzzers and consume less power, but they are not as loud. They can operate on a DC or AC voltage.

- To use a buzzer with an ESP32, you would connect the buzzer’s positive lead to one of the ESP32’s digital output pins and the negative lead to a ground pin. Then, using code, you can configure the output pin to be high or low to turn the buzzer on or off.

FEATURES

• Black in colour

• With internal drive circuit

• Sealed structure

• Wave solderable and washable

APPLICATIONS

• Computer and peripherals

• Communications equipment

• Portable equipment

• Automobile electronics

• POS system

• Electronic cash register

2.2.5 OLED DISPLAY

An OLED (Organic Light-Emitting Diode) display is a type of display that uses organic compounds to emit light when an electric current is applied. It is a self-emitting display that does not require a backlight and offers high contrast and low power consumption. OLED displays are thin, lightweight, and flexible, making them ideal for use in wearable devices and other portable electronics.

Figure 2.2.5 OLED Display

FEATURES

- Display Size:   0.96 inch

- Dimension:   26.3x26.1 mm

- Display area:   21.7x10.9 mm

- Display Driver IC:   SSD1315

- Resolution： 128x64

- Port:   I2C

- Operating Voltage:   3.3-5V

- Display colour:   Blue, White (Optional)

ADVANTAGES

- Thin and Lightweight

- High Contrast and Brightness

- Low Power Consumption

- Fast Response Time

- Wide View Angle

- No Backlight Required

- Environmentally Friendly

CHAPTER- 3

3.1 CIRCUIT DIAGRAM

- FIG 3.1 CIRCUIT DIAGRAM

3.2 INTERFACING DHT 22 WITH ESP32

3.2.1 PIN DESCRIPTION

| Pin of DHT11 | Description |
| --- | --- |
| VCC | Power supply |
| DATA | Input/output data |
| GND | ground |

- FIG 3.2 INTERFACING ESP 32 WITH DHT 22

3.3 INTERFACING GAS SENSOR WITH ESP 32

Fig 3.3 Interfacing MQ-2 with ESP32

MQ-2 sensor is an easy-to-use gas sensor which can give output in both analog and digital. Digital output only gives HIGH or LOW value indicating gas detection however here we will be using analog output which gives more detailed reading and helps to note gas level.

Analog pin output is proportional to gas concentration more is the gas available higher is the analog output value. It is important to note that the MQ-2 sensor has Op Amp with a high precision comparator (LN393) which takes the analog signal and digitizes it to be available at digital output of the sensor.

MQ-2 sensors can detect gas concentration ranging from 200 ppm to 10000 ppm. Here ppm denotes Parts-per-million which is a unit to indicate concentration of gas.

3.4 INTERFACING BUZZER WITH ESP 32

Fig 3.4 Interfacing Buzzer with ESP32

To interface an ESP32 with a buzzer, you'll need to follow these steps:

Step 1: Gather the Required Components,ESP32 development boardActive or passive buzzer (depending on the type you have).Breadboard and jumper wires

Step 2: Connect the Buzzer to the ESP32.Connect the positive terminal of the buzzer to any GPIO pin of the ESP32. For example, let's use GPIO 12.Connect the negative terminal of the buzzer to a ground (GND) pin on the ESP32.

Step 3: Code the ESP32 to Control the Buzzer.Set up the ESP32 development environment using the Arduino IDE or any other preferred platform.

Write the code to control the buzzer. Here's a sample code snippet using the Arduino IDE

3.5 INTERFACING OLED WITH ESP 32

Fig 3.5 interfacing oled with esp 32

- GND Ground connection for the module. Connect to the GND pin of the ESP32 Devkit.

- VCC Provides power for the module. Connect to the 3.3V pin of the ESP32 Devkit.

- SCL Serial Clock pin. Used for providing clock pulse for I2C Communication.

- SDA Serial Data pin. Used for transferring Data through I2C communication.

- CHAPTER - 4

- FIRE BASE AND ITS RESULT

- 4.1 FIREBASE

- Firebase is a toolset to “build, improve, and grow your app”, and the tools it gives you cover a large portion of the services that developers would normally have to build themselves but don’t really want to build because they’d rather be focusing on the app experience itself. This includes things like analytics, authentication, databases, configuration, file storage, push messaging, and the list goes on. The services are hosted in the cloud and scale with little to no effort on the part of the developer.”

- Fig 4.1.1(a) firebase

- 4.2 STEPS TO ADD FIRE BASE TO THE SENSOR DATA

- 1.Create a New Project

- Follow the next instructions to create a new project on Firebase.

- Go to Firebase and sign in using a Google Account.

- Click Get Started, and then Add project to create a new project.

- Give a name to your project, for example: ESP32 Firebase Demo.

- Fig 4.2.1(a)project name

- 2. Set Authentication Methods

- You need to set authentication methods for your app.

- “Most apps need to know the identity of a user. In other words, it takes care of logging in and identify the users (in this case, the ESP32). Knowing a user’s identity allows an app to securely save user data in the cloud and provide the same personalized experience across all of the user’s devices.”

- Fig 4.2.1(b) Authentication

- 3. Creating a Realtime Database

- The next step is creating a Realtime Database for your project. Follow the next steps to create the database.

- Fig 4.2.1© Realtime Database

- Your database is now created. You need to copy and save the database URL—highlighted in the following image—because you’ll need it later in your ESP32 code.

- Fig 4.2.1 (d) URL

- Copy the API Key to a safe place because you’ll need it later.

- Fig 4.2.1(e)  authentication key

- 4.3 ESP32 Store Data to Firebase Database

Fig 4.3 conversion

4.4 KODULAR COMPANION

- The Kodular rapid mobile app development utility is used for mobile app development.

- Kodular follows a drag and drop approach for app development and there is no need to write any code. The utility as of now only supports the Android OS – so apps developed using the Kodular framework will work only for Android phones. For dynamic testing of the mobile app .while it is under development, the Kodular Companion app is provided. This needs to be downloaded and installed on your mobile phone. For the Kodular app to be transferred from the laptop to the mobile phone both the devices will need to be connected to the same WiFi network. Under the Test tab in Kodular, there is a Connect to Companion option, which will open up a 2D QR code. This will be scanned by the Kodular Companion app

- Fig 4.4 kodular

- The utility has two modes:

- ● Developer mode

- ● Blocks mode

- The Developer mode will help define the look and feel of the mobile app. In the Developer mode, there are different functional components available – User Interface, Sensors, Connectivity, Firebase, and so on.

- Create a Firebase account or login if you already have one. In the console, click on add project, give your project a name and select your country/region. Click on "Create Project" button. Once your project is created, click on the "Web" button to add a new web app to your project. Give a name to your app. Copy the Firebase configuration details from the next screen. These details will include the Firebase API key, authentication domain, database URL, project ID, storage bucket, and messaging sender ID.Now, go to Kodular and create a new project. From the designer screen, click on the "Connectivity" menu and then select "Firebase Authentication".Enter your Firebase API key and authentication domain in the appropriate fields.Click on the "Connect" button to verify your connection.Next, go to the "Connectivity" menu again and select "Realtime DB".

- Enter your database URL and project ID in the appropriate fields.Click on the "Connect" button

- to verify your connection.

- Fig 4.4.1 kodular web page

- ARDUINO CODE

- #include "DHT.h"

- #include<Wire.h>

- #include<Adafruit_GFX.h>

- #include<Adafruit_SSD1306.h>

- #include <Arduino.h>

- #include <WiFi.h>

- #include <Firebase_ESP_Client.h>

- #include "addons/TokenHelper.h"

- #include "addons/RTDBHelper.h"

- #define SCREEN_WIDTH 128

- #define SCREEN_HEIGHT 64

- #define OLED_RESET -1

- int LED = 2;

- int Sensor_input = 25;

- int buz = 13;

- #define DHTPIN 14

- #define DHTTYPE DHT22

- DHT dht(DHTPIN, DHTTYPE);

- Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

- #define WIFI_SSID "Redmi Note 9 Pro Max"

- #define WIFI_PASSWORD "Tharun..."

- //firebase stuff

- #define API_KEY "AIzaSyBhY7dYGCYXJBSzt-0CTPF0xB6oMCfwyXc"

- #define DATABASE_URL "https://smart-helmet-733ce-default-rtdb.asia-southeast1.firebasedatabase.app/"

- FirebaseData fbdo;

- FirebaseAuth auth;

- FirebaseConfig config;

- unsigned long sendDataPrevMillis = 0;

- bool signupOK = false;

- float h;

- float t;

- float f;

- float hic;

- float hif;

- int sensor_Aout;

- void setup()

- {

- Serial.begin(115200);

- if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C))

- {

- Serial.println(F("SSD1306 allocation failed"));

- for(;;);

- }

- display.display();

- delay(2000);

- display.clearDisplay();

- display.setCursor(10,10);

- display.setTextSize(1);

- display.setTextColor(WHITE);

- display.println("L2-N0");

- display.display();

- delay(2000);

- display.clearDisplay();

- Serial.begin(115200);

- Serial.println(F("DHTxx test!"));

- pinMode(LED, OUTPUT);

- pinMode(buz,OUTPUT);

- dht.begin();

- wifiandfirebaseinit();

- }

- void loop() {

- delay(2000);

- h = dht.readHumidity();

- t = dht.readTemperature();

- f = dht.readTemperature(true);

- if (isnan(h) || isnan(t) || isnan(f)) {

- Serial.println(F("Failed to read from DHT sensor!"));

- return;

- }

- hif = dht.computeHeatIndex(f, h);

- hic = dht.computeHeatIndex(t, h, false);

- Serial.print(F("Humidity: "));

- Serial.print(h);

- Serial.print(F("%  Temperature: "));

- Serial.print(t);

- Serial.print(F("°C "));

- Serial.print(f);

- Serial.print(F("°F  Heat index: "));

- Serial.print(hic);

- Serial.print(F("°C "));

- Serial.print(hif);

- Serial.println(F("°F"));

- sensor_Aout = analogRead(Sensor_input);  //Analog value read function/

- Serial.print("Gas Sensor: ");

- Serial.print(sensor_Aout);   /Read value printed/

- Serial.print("\t");

- Serial.print("\t");

- display.clearDisplay();

- display.setCursor(10,10);

- display.setTextSize(2);

- display.setTextColor(WHITE);

- display.print("Humidity:");

- display.println(h);

- display.display();

- delay(1500);

- display.clearDisplay();

- display.setCursor(10,10);

- display.setTextSize(2);

- display.setTextColor(WHITE);

- display.println("Temperature:");

- display.println(t);

- display.display();

- delay(1500);

- display.clearDisplay();

- display.setCursor(10,10);

- display.setTextSize(2);

- display.setTextColor(WHITE);

- display.println("Gas Value");

- display.print(sensor_Aout);

- display.display();

- delay(1500);

- if (sensor_Aout > 1800|| t>50) {    //if condition with threshold 1800/

- Serial.println("Gas Leaked");

- digitalWrite(buz, HIGH);

- digitalWrite (LED, HIGH) ; /*LED set HIGH if Gas detected */

- display.clearDisplay();

- display.setCursor(10,10);

- display.setTextSize(2);

- display.setTextColor(WHITE);

- display.println("GASLEAKED");

- display.display();

- delay(1000);

- }

- else {

- Serial.println("No Gas Leaked");

- digitalWrite(buz,LOW);

- digitalWrite (LED, LOW) ;  /*LED set LOW if NO Gas detected */

- }

- firebasesend();

- }

- void wifiandfirebaseinit(void)

- {

- WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

- Serial.print("Connecting to Wi-Fi");

- while (WiFi.status() != WL_CONNECTED)

- {

- Serial.print(".");

- delay(300);

- }

- Serial.println();

- Serial.print("Connected with IP: ");

- Serial.println(WiFi.localIP());

- Serial.println();

- config.api_key = API_KEY;

- config.database_url = DATABASE_URL;

- if (Firebase.signUp(&config, &auth, "", ""))

- {

- Serial.println("ok");

- signupOK = true;

- }

- else

- {

- Serial.printf("%s\n", config.signer.signupError.message.c_str());

- }

- config.token_status_callback = tokenStatusCallback;

- Firebase.begin(&config, &auth);

- Firebase.reconnectWiFi(true);

- }

- void firebasesend(void)

- {

- if (Firebase.ready() && signupOK && (millis() - sendDataPrevMillis > 15000 || sendDataPrevMillis == 0))

- {

- sendDataPrevMillis = millis();

- Firebase.RTDB.setFloat(&fbdo, "test/Humidity", h);

- Firebase.RTDB.setFloat(&fbdo, "test/Temperature", t);

- Firebase.RTDB.setInt(&fbdo, "test/Gasvalue", sensor_Aout);

- Firebase.RTDB.setFloat(&fbdo, "test/HeatIndex", hic);

- }

- }

- RESULT

- FIG 4.5 KODULAR RESULT

- CHAPTER - 5

- FUTURE SCOPE OF THE PROJECT

- 5.1 FUTURE SCOPE

- The future scope for smart helmets for coal miners is vast and holds potential for further advancements in safety, monitoring, and communication. Here are some potential areas of future development:

- Integration of advanced sensors: Future smart helmets can incorporate more advanced sensors to monitor various parameters. These may include sensors for air quality, particulate matter, toxic gases, and radiation levels. Additional sensors can provide miners with more comprehensive real-time information about their surroundings, enabling them to make informed decisions and take appropriate safety measures. By integrating artificial intelligence (AI) and machine learning algorithms, smart helmets can analyze data collected from various sensors to detect patterns, identify hazards, and predict potential safety risks. AI can help in real-time monitoring, early warning systems, and proactive safety measures for miners.

- Augmented reality (AR) and heads-up display (HUD): Future smart helmets can feature AR capabilities and HUDs, allowing miners to overlay relevant information directly onto their field of view. This can include real-time sensor data, navigation assistance, instructions, and emergency alerts. AR can enhance situational awareness and improve decision-making in complex and hazardous mining environments.

- Communication and collaboration features: Smart helmets can be further enhanced with advanced communication and collaboration capabilities. Integration with robust wireless networks, voice recognition, and advanced audio systems can facilitate seamless communication between miners, control centers, and emergency response teams. Real-time video streaming or image sharing can also enable remote assistance and collaboration among miners.

- Data analytics and cloud integration: Smart helmets can leverage cloud computing and data analytics to process and analyze large volumes of sensor data. Cloud integration can enable centralized data storage, remote monitoring, and data-driven insights for mine operators and safety managers. The analysis of historical data can help identify trends, improve safety protocols, and optimize mine operations.

- Intelligent lighting and visibility: Smart helmets can integrate intelligent lighting systems that adjust brightness and color temperature based on the surrounding environment. This can improve visibility in low-light conditions, reduce eye strain, and enhance overall safety and comfort for miners.

- Regulatory compliance and industry standards: As the technology evolves, future smart helmets for coal miners should align with regulatory standards and industry guidelines. Collaborating with safety organizations, mining associations, and regulatory bodies will ensure that the smart helmets meet the required safety standards and are adopted effectively.

- The future scope for smart helmets in the coal mining industry is promising, with advancements in sensor technology, connectivity, artificial intelligence, and data analytics. These developments have the potential to significantly enhance the safety, monitoring, and well-being of coal miners, contributing to a safer and more efficient mining environment.

- 5.2 CONCLUSION

- By incorporating these sensors and the ESP32 microcontroller, the smart helmet can detect and monitor multiple environmental factors in real-time. The MQ2 gas sensor allows for the detection of potentially dangerous gas levels, ensuring the safety of the wearer. The KY-026 flame sensor enables the identification of fire or flames, providing early warning and enabling prompt action.Additionally, the DHT11 temperature and humidity sensor offers continuous monitoring of these parameters, providing essential information for maintaining optimal conditions and ensuring the well-being of the wearer.

- The integration of the buzzer further enhances the safety features of the smart helmet. It can be activated when dangerous gas levels or flames are detected, alerting the wearer to potential hazards and facilitating quick response and evacuation.

- With the ESP32 microcontroller, these sensors can be easily interfaced and controlled, allowing for efficient data processing, decision-making, and communication. The microcontroller facilitates real-time monitoring and enables the transmission of data to a central control center for remote monitoring and analysis.

- Overall, the smart helmet project using the MQ2, KY-026, DHT11, and buzzer, combined with the ESP32 microcontroller, offers an advanced safety solution that enhances protection, reduces risks, and improves situational awareness for the wearer in various environments such as mining, construction, industrial settings, and more.

- PROJECT OUTCOMES VS COURSE OUTCOMES

| Course outcomes | Formulate hypothesis for the problem statement with sound technical knowledge from selected project domain. (CO1) | Design Engineering Solution to the problem statement with systematic approach. (CO2) | Analyse and develop an efficient solution for implementation of the project. (CO3) | Apply the theoretical concepts while providing solution to the problem statement with teamwork and multidisciplinary approach. (CO4) | Demonstrate professionalism with ethics while preparing and presenting the project work. (CO5) |
| --- | --- | --- | --- | --- | --- |
| Project outcomes |  |  |  |  |  |
| To understand the concept of Smar tHelmet for CoalMiners from the existing literature and build problem statement for the proposed project | X |  | X | X | X |
| To implement a mechanism to record and capture the health data from various sensors in secure manner | X | X | X | X |  |
| To implement techniques for Smart Helmet for coal miners including DHT22, MQ2 and understand the performance of sensors that determine the accuracy and efficiency |  | X | X | X |  |
| To implement modified Arduino programs and present a comparative analysis with various techniques | X | X | X |  |  |
| Apply the best algorithm and various techniques for sensing humidity, temperature and GAs |  | X | X |  |  |